# ASIC Area and Power Estimation Report

SoC: 4-Thread SIMT GPU with 4x4 BF16 Tensor Core

EE 533 - Network Processor Design

Raymond | Team 5

# 1. Methodology

This report estimates the ASIC area, power, and gate count of the SoC design using open-source EDA tools and standard cell libraries. The methodology:

1. RTL Source: Jeremy Cai's simulation RTL (Verilog), identical to the design verified in iverilog simulation and synthesized to Xilinx Virtex-II Pro FPGA.

2. Synthesis Tool: Yosys 0.38 (open-source logic synthesis)

3. Target Library: Nangate 45nm Open Cell Library (FreePDK45-compatible, published cell areas in um2)

4. Scaling: Published TSMC density scaling factors for 28nm, 16nm, 7nm, 5nm nodes

5. Memory: SRAM area estimated separately using published bit-cell densities per node

# 2. Design Hierarchy

soc (top)
  +-- cpu_mt             4-thread FGMT barrel ARM CPU
  |     +-- alu, barrel_shifter, bdtu, cu, regfile
  +-- cp10_regfile        Coprocessor registers (10 CRs)
  +-- dma_engine          DMA controller (D_IMEM/D_UNPACK/D_PACK)
  +-- pkt_proc            Packet processor (command FSM)
  +-- conv_fifo           Convertible FIFO (RX/TX/SRAM modes)
  +-- sm_core             SIMT GPU streaming multiprocessor
  |     +-- sp_core x4    4 streaming processors
  |     |     +-- gpr_regfile (16x16b), pred_regfile
  |     |     +-- int16alu, bf16fpu, pplbfintcvt
  |     +-- tc_top        Tensor core wrapper
  |     |     +-- tensor_core -> bf16sa (4x4 systolic array)
  |     |           +-- bf16pe x16 (BF16 multiply-accumulate)
  |     +-- fetch_unit, sm_decoder, scoreboard, simt_stack
  +-- test_i_mem          CPU IMEM (4096x32 SRAM)
  +-- test_d_mem          CPU DMEM (4096x32 SRAM)
  +-- test_gpu_imem       GPU IMEM (256x32 SRAM)
  +-- test_gpu_dmem x4    GPU DMEM banks (1024x16 each)
  +-- test_dpfifo         FIFO SRAM (4096x64)

# 3. Synthesis Results (Nangate 45nm)

## 3.1 Overall Summary

| Metric | Value |
| --- | --- |
| Total standard cells | 60,518 |
| Combinational cells | 52,744 (87.2%) |
| Sequential cells (flip-flops) | 7,774 (12.8%) |
| Memory blocks (SRAM) | 24 (not mapped to std cells) |
| Logic chip area | 103,856 um2 = 0.104 mm2 |
| SRAM bits | 598,016 bits |

## 3.2 Cell Distribution by Category

| Category | Count | % | Purpose |
| --- | --- | --- | --- |
| NAND gates | 8,431 | 13.9% | Core logic |
| AOI (AND-OR-Invert) | 7,883 | 13.0% | Compound logic in adders/comparators |
| Flip-flops (DFF) | 7,774 | 12.8% | Pipeline registers, FSM state |
| MUX (2:1) | 7,279 | 12.0% | Data routing, RF muxes, thread select |
| OAI (OR-AND-Invert) | 6,649 | 11.0% | Compound logic in BF16 arithmetic |
| NOR gates | 6,399 | 10.6% | Core logic |
| XOR/XNOR gates | 6,295 | 10.4% | Adders, BF16 exponent/mantissa ops |
| Inverters | 4,117 | 6.8% | Signal inversion |
| AND gates | 3,539 | 5.8% | Glue logic, enables |
| OR gates | 2,152 | 3.6% | Glue logic |

## 3.3 Top 10 Individual Cell Types

| Cell | Count | % |
| --- | --- | --- |
| MUX2_X1 (2:1 mux) | 7,279 | 12.0% |
| DFFR_X1 (DFF w/ reset) | 6,622 | 10.9% |
| NAND2_X1 | 6,286 | 10.4% |
| OAI21_X1 | 4,578 | 7.6% |
| NOR2_X1 | 4,517 | 7.5% |
| AOI21_X1 | 4,134 | 6.8% |
| INV_X1 | 4,117 | 6.8% |
| XNOR2_X1 | 3,692 | 6.1% |
| XOR2_X1 | 2,603 | 4.3% |
| AND2_X1 | 2,561 | 4.2% |

MUX2 is the most common cell, reflecting the heavy data multiplexing in the 4-thread SIMT architecture (register file read ports, forwarding paths, tensor core gather/scatter, DMA bank selection).

# 4. Area Breakdown by Function

## 4.1 Logic Area (from Yosys cell mapping)

| Component | Area (um2) | % of Logic |
| --- | --- | --- |
| ARM CPU (4-thread barrel) | 19,782 | 31.3% |
| GPU SM Core (4x SP lanes) | 14,298 | 22.6% |
| GPU Tensor Core (4x4 SA) | 9,686 | 15.3% |
| Packet Processor + FIFO | 6,470 | 10.2% |
| CP10 + DMA Engine | 3,888 | 6.2% |
| Other (memory wrappers, etc) | 9,063 | 14.3% |
| TOTAL | 63,186 | 100% |

## 4.2 SRAM Area (estimated from bit-cell density)

| Memory | Size | Bits | Area @ 45nm (um2) |
| --- | --- | --- | --- |
| CPU IMEM | 4096 x 32 | 131,072 | 65,536 |
| CPU DMEM | 4096 x 32 | 131,072 | 65,536 |
| GPU IMEM | 256 x 32 | 8,192 | 4,096 |
| GPU DMEM x4 | 4 x 1024 x 16 | 65,536 | 32,768 |
| Conv FIFO | 4096 x 64 | 262,144 | 131,072 |
| TOTAL |  | 598,016 | 299,008 |

SRAM density: 0.5 um2/bit at 45nm (6T SRAM). SRAM dominates total area (~83%).

## 4.3 Total Area at 45nm

| Component | Area (um2) | Area (mm2) | % |
| --- | --- | --- | --- |
| Logic (std cells) | 63,186 | 0.063 | 17% |
| SRAM | 299,008 | 0.299 | 83% |
| TOTAL | 362,194 | 0.362 | 100% |

# 5. TSMC Node Scaling

Area estimates scaled using published TSMC logic density and SRAM bit-cell size:

Area Scaling to TSMC Nodes

| Node | Logic (mm2) | SRAM (mm2) | Total (mm2) | vs 45nm |
| --- | --- | --- | --- | --- |
| 45nm | 0.063 | 0.299 | 0.362 | 1.00x |
| TSMC 28nm | 0.028 | 0.072 | 0.100 | 0.28x |
| TSMC 16nm | 0.014 | 0.042 | 0.056 | 0.15x |
| TSMC 7nm | 0.006 | 0.024 | 0.030 | 0.08x |
| TSMC 5nm | 0.004 | 0.013 | 0.017 | 0.05x |

Note on FPGA comparison: The Virtex-II Pro XC2VP50 full die is 365 mm2, but most of that is programmable routing fabric (~55%), configuration SRAM (~15%), I/O ring (~10%), and unused PowerPC cores (~5%). The actual CLB/BRAM/MULT resources occupy ~55 mm2 (15% of die). Our design uses 89% of slices, so the effective FPGA silicon for our logic is ~49 mm2. The ASIC at 45nm (0.36 mm2) is ~136x smaller than this effective area. At TSMC 7nm (0.03 mm2), the ratio increases to ~1,600x.

# 6. Power Estimation

Dynamic power estimated from cell count and published power-per-gate metrics:

Power Estimation by Node

| Node | Frequency | Vdd | Logic Power | Notes |
| --- | --- | --- | --- | --- |
| 45nm | 125 MHz | 1.1V | ~1,150 mW | FPGA clock rate |
| TSMC 28nm | 500 MHz | 0.9V | ~1,080 mW | 4x faster, lower V |
| TSMC 16nm | 800 MHz | 0.8V | ~580 mW | FinFET benefit |
| TSMC 7nm | 1 GHz | 0.75V | ~260 mW | High performance |
| TSMC 5nm | 1.2 GHz | 0.7V | ~180 mW | Mobile-class |

Note: SRAM leakage and dynamic power add ~50-200 mW depending on access patterns and node. Total system power at TSMC 7nm: estimated 300-500 mW at 1 GHz.

# 7. FPGA vs ASIC Comparison

The Virtex-II Pro die is 365 mm2, but this includes routing fabric, configuration SRAM, I/O ring, and unused PowerPC cores. A fair comparison uses only the effective silicon area occupied by our logic.

FPGA (Virtex-II Pro) vs ASIC Comparison

| Metric | FPGA (XC2VP50) | ASIC (45nm) | ASIC (7nm) |
| --- | --- | --- | --- |
| Full die area | 365 mm2 | 0.36 mm2 | 0.03 mm2 |
| Effective logic area | ~49 mm2 (89% of CLBs) | 0.36 mm2 | 0.03 mm2 |
| Area ratio (vs FPGA effective) | 1x | 136x smaller | 1,600x smaller |
| Clock Frequency | 125 MHz | ~200 MHz | ~1 GHz |
| Logic Power | ~12 W (full board) | ~1.2 W | ~0.3 W |
| Process Node | 130nm | 45nm | 7nm |
| Slices / Std Cells | 21,060 slices | 60,518 cells | 60,518 cells |
| Block RAM / SRAM | 145 RAMB16 | 598 Kbit | 598 Kbit |
| Multipliers | 24 MULT18X18 | In std cells | In std cells |

FPGA area breakdown: ~55% routing fabric, ~15% config SRAM, ~15% CLB/BRAM/MULT, ~10% I/O ring, ~5% PowerPC cores (unused). Only the CLB/BRAM/MULT fraction (~55 mm2) is functionally equivalent to the ASIC logic + SRAM.

# 8. Design Efficiency Analysis

Compute density (TOPS/mm2 for BF16 tensor core):

Tensor core: 4x4 BF16 matmul = 128 FLOPs per WMMA (64 MACs x 2 FLOPs)
WMMA latency: ~44 cycles

At 1 GHz (TSMC 7nm):
  Throughput = 128 / 44 = 2.9 GFLOPS (BF16)
  Tensor core area = 0.001 mm2 (scaled from 9,686 um2 at 45nm)
  Compute density = 2.9 / 0.001 = 2,900 GFLOPS/mm2

For comparison:
  NVIDIA A100 tensor cores: ~600 GFLOPS/mm2 (BF16)
  Our design is ~5x denser per tensor core,
  but A100 has 432 tensor cores vs our 1.

# 9. Methodology Notes

Limitations of this estimation:

1. Yosys uses generic ABC technology mapping, not commercial synthesis tools (Synopsys DC / Cadence Genus). Commercial tools achieve 10-20% better area.

2. Nangate 45nm is an academic library, not a production TSMC library. Cell areas are representative but not exact.

3. SRAM area is estimated from published bit-cell densities, not from actual SRAM compiler output.

4. Power estimation uses rule-of-thumb scaling, not gate-level power analysis. Actual power depends on switching activity, clock tree, and leakage.

5. No physical design (placement, routing, clock tree) was performed. Routing overhead typically adds 20-40% to cell area.

6. Memory blocks ($mem_v2) are not mapped to standard cells — they would be replaced by SRAM macros in a real ASIC flow.

# 10. Conclusion

The SoC design containing a 4-thread SIMT GPU with 4x4 BF16 tensor core, barrel ARM CPU, DMA engine, and packet processor synthesizes to approximately 60,500 standard cells (250K gate equivalents). At TSMC 28nm, the total die area including SRAM is estimated at 0.10 mm2, consuming approximately 1W at 500 MHz. The design is dominated by SRAM (83% of area) and the ARM CPU (31% of logic). The tensor core, despite its compute capability, accounts for only 15% of logic area — demonstrating efficient hardware utilization for BF16 matrix operations.